[image: image1.png]


Professional WDR Box camera
INSTRUCTION MANUAL

                                                        Power option
                                         ­12VDC
                                      ­12VDC / AC24V(Dual Power)
[image: image60.jpg]



Dear Customers!
By selecting this product, you have decided to use a professional device that guarantees highest quality and reliability. We would like to thank you very much for your confidence and kindly ask you to read the following instructions carefully before installation and operation in order to take full advantage of all quality features regarding this product.   

[image: image2.png]CAUTION

RISK OF ELECTRIC

SHOCK
DO NOT OPEN

CAUTION

TO REDUCE THE RISK OF ELECTRIC SHOCK,
DO NOT REMOVE THE COVER (OR BACK).
NO USER SERVICEABLE PARTS INSIDE.
REFER SERVICING TO QUALIFIED PERSONNEL.





[image: image9.png]


The lighting flash with an arrowhead symbol, within an equilateral 
triangle is intended to alert the user to the presence of uninsulated 
dangerous voltage within the product’s enclosure that may be of 
sufficient magnitude to constitute a risk of electric shock to persons.
[image: image10.png]


The exclamation point within an equilateral triangle is intended to alert 
the user to the presence of important operating and maintenance 
(servicing) instructions in the literature accompanying the appliance.
INFORMATION
This equipment has been tested and found to comply with limits for a Class A digital device, pursuant to part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference when the equipment is operated in a commercial environment. This equipment generates, uses, and can radiate radio frequency energy and, if not installed and used in accordance with the instruction manual, may cause harmful interference to radio communications. Operation of this equipment in a residential area is likely to cause harmful interference in which case the user will be required to correct the interference at its own expense.
WARNING
Changes or modifications not expressly approved by the manufacturer could void the user’s authority to operate the equipment.
CAUTION – To prevent electric shock and risk of the fire hazards
-Do NOT use power source other than that specified.
   -Do NOT expose this appliance to rain or moisture.
This installation should be made by a qualified service person and should conform to 
all local codes.
Table of content
1. Precautions -------------------------------------------------------------------4 
2. Limitation of liability ---------------------------------------------------------5
3. Disclaimer of warranty -------------------------------------------------------5
4. Package ----------------------------------------------------------------------6
5. Name and function of each part------------------------------------------7~8
6. Special features of the model------------------------------------------------9
7. Installation---------------------------------------------------------------10~12
8. Functions and operation -----------------------------------------------13~22
9. Troubleshooting ---------------------------------------------------------23~24
10. Dimension (mm) -----------------------------------------------------------24
11. Specification----------------------------------------------------------------25
 1. Precautions

· Please read the manual carefully before the installation in order to set up the camera correctly and to obtain the best picture quality.

· Please keep the manual in good condition for your future reference and service application.

· Installation and services should be only carried out by an authorized personnel according to local safety regulations.

· If any liquid or solid matter gets into the housing, immediately disconnect the camera from power supply and have it checked by your authorized dealer before reusing.

· Avoid installing the camera at extremely hot or cold places.

· If you are not a certified person, never try to dismantle the camera. To avoid electric shock, never remove the screws or covers. There are no parts inside that need maintenance by the user. All maintenance should be carried out by qualified personnel.

· Avoid installing the camera at a place of high humidity.

· Avoid installing the camera at the place exposed to gas or oil. 

· Keep the top glass of the lens always clean in order to obtain the best picture quality all the time. Be careful not to be stained by fingerprint.

· Don't face the camera directly toward sunlight or sunlight reflecting area. 

CCD may go defective at this condition.

· Please give a special attention to keep the unit from dangerous drop or external shock during
 the process of transportation or handling. 

· Never try to touch the camera in wet hand. It may cause an electric shock.

· Do not expose the camera to radioactivity. It causes a serious damage on the CCD. 

2. Limitation of liability

This publication is provided “AS IS” without warranty of any kind, either express or implied, including but not limited to, the implied warranties of merchantability, fitness for any particular purpose, or noninfringement of the third party's right. This publication could include technical inaccuracies or typographical errors. Changes are added to the information herein, at any time, for the improvements of this publication and/or the corresponding product(s).

3. Disclaimer of warranty

In no event shall seller be liable to any party or any person, except for replacement or reasonable maintenance of the product, for the cases, including but not limited to below :

(1) Any damage and loss, including without limitation, direct or indirect, special,  

    consequential or exemplary, arising out of or relating to the product;

(2) Personal injury or any damage caused by inappropriate use or negligent operation 

    of the user; 

(3) Unauthorized disassemble, repair or modification of the product by the user;

(4) Inconvenience or any loss arising when images are not displayed, due to any reason 

    or cause including any failure or problem of the product;

(5) Any problem, consequential inconvenience, or loss or damage, arising out of the 

    system combined by the devices of third party.

(6) Any claim or action for damages, brought by any person or organization being 

    photogenic subject, due to violation of privacy with the result of that surveillance-

    camera's picture, including saved data, for some reason, becomes public or is used 

    for the purpose other than surveillance.

 4.Package     
[image: image3.png]=



                           [image: image4.png]



Professional  
WDR Camera                            Instruction manual                 
1EA                                            1EA
[image: image11.jpg]


[image: image12.jpg]


                           
C-Mount Adaptor                                 Iris Jack
1EA                                             1EA
5. Name and function of each part
5-1. Front                     [image: image13.emf]
5-2. Right side      

[image: image14.emf][image: image15.emf]
[image: image16.emf][image: image17.emf][image: image18.emf][image: image19.emf]
[image: image20.emf]
[image: image21.emf]
[image: image22.jpg]<CAMERA NAME>

ABCDEF@HIJKLM
NOPQRSTUVWXYZ
abcdefghijklim

nopqrstuvwxyz
-. 123456789

M CLR [ISBEND




[image: image23.jpg]BACK YARD





[image: image24.jpg]<CAMERA NAME>

ABCDEFGHIJKLM
NOPQRSTUVWXYZ
abcdefghijklim

nopqrstuvwxyz
-. 0123456789

—— CLR POS END
8





[image: image25.emf][image: image26.emf][image: image27.emf]
- Back focus adjustment Knob: Fine tune the Back Focal Length

5-3. Top
[image: image28.emf] SHAPE  \* MERGEFORMAT 



5-4. Rear side
[image: image29.jpg]<MAIN MENU>

. LENS DC IRIS4
. EXPOSURE &

. DAY/NIGHT AUTOI
.WDR/BLC/ECLPS ECLPS 4
.WHITE BAL ATW
-2D-DNR DISABLED
.3D-DNR ONS

. EFFECTS &

. SPECIAL 4

0. EXIT RET 4

|
2
3
4
H
6
7
> 8
9
I

<EFFECTSA MENU>

»DZOOM MODE OFF
IMAGE FREEZE OFF
D-EFFECT OFF
CONTRAST 50
SHARPNESS 50
COLOR ADJUST 4
TARGET CROSS
BAD PIXEL <
INITIAL SETJ
SAVE





[image: image30.emf]
[image: image31.emf]
[image: image32.emf]
[image: image33.jpg]ECLIPSE MASKING AREA

ECLIPSE ON ECLIPSE OFF



[image: image34.emf][image: image35.emf][image: image36.jpg]


[image: image37.emf]
[image: image38.emf]
[image: image39.emf]
- Power indicator : Red LED ON when camera’s working .

- OSD function control key
- VIDEO OUTPUT : BNC-Fe.

- POWER INPUT : Power input terminal.

6. Special features of the model
1. Higher resolution of 600TVL(D)/650TVL(N)
This unit has come up with an innovative way of increasing cubic resolution to more than 30% higher than that of conventional 600TVL cameras.
It upgrades horizontal resolution to 600TVL and vertical resolution to 540TVL so that the cubic resolution becomes 324KTVL while conventional cameras provide only 240KTVL.
2. Unbeatable WDR Performance
This unit provides a very unique and far superior image scanning capability of defining the details of differently contrasted areas compared with any of conventional WDR camera. With current WDR technologies, defining images in shaded areas are typically not clear enough with over-exposed picture quality. This camera provides a special resolution to intelligently analyze the exposure ratio and to optimize it into an unbeatably clear image quality.
3. Super Low Light sensitivity
The system specifies itself as the best resolution in the market that overcomes the limitation of compromising light sensitivity from conventional WDR cameras. 
With its special technology, it provides remarkable low light sensitivity of 0.1Lux /F2.0 ~ 0.0001Lux w /Sens-up 512x which is at least 3 times better than any of conventional WDR cameras.
4. 2D/3D Noise Reduction
2D/3D DNR achievement is another reference in new WDR technology. 
It reproduces noiseless images with 2D/3D filtering Noise Reduction Technology so that users can save the volume of storage media and HDD capacity in DVR. 
2D DNR helps preserved edged in the picture, while 3D DNR realizes clear images with no ghost effect.
5. DIS (Digital Image Stabilizer)
This technology reduces blurring associated with the motion of the subject by using pixels outside the border of the visible frame to provide a buffer for the motion. 
Increasing the exposure time without blurring the image makes the moving subject stabilized digitally.
6. Anti-Color Rolling
When operation a wide dynamic range camera in an environment with a fluorescent lighting, an effect that is called “color rolling” usually happens due to the lack of the synchronization between the camera and the neon light.
New WDR technology tracks the light changes relative to the camera and adapts its own timing to the periodicity of fluorescent illumination to the sample light at a constant phase, virtually elimination the color rolling effect.
7. Installation

7-1. Lens
Lens is not supplied with the unit. C-Mount or CS-Mount type of lens is useable.

Please select a lens that has a good F-Stop number in order to get the best picture quality. Use of DC auto iris lens is recommended to achieve the best results for operating this product effectively.

☞ Caution
Please make sure the surface of the lens be always clear. Clean dirt or water drops with soft cloth in order to avoid the picture blooming or reflection.

7-2. When using an auto iris lens (DC-drive)
1. Please take off the sleeve outer cable of the lens by 10mm

2. Please take off the sleeve inner cable of the lens by 2mm. 

[image: image40.emf][image: image6.jpg]- ~ - =

z N Y ~
/ Approx. 10mm \ , Approx. 2mm \
- v~
B ! s !
W= ==
S 8 PN’
N 7/ 10mm  ,
~ - s -




	Pin No.
	Lens

	
	DC

	No.1 Pin
	Damping -

	No.1 Pin
	Damping +

	No.1 Pin
	Drive +

	No.1 Pin
	Drive -


3. Solder the wires on the pin-connector (iris-jack) supplied with the lens
 according to the pin configuration as shown in the drawing. 

4. Assemble the lens. 

5. Connect the Auto Iris jack.
6. Adjust the DIP S/W to DC according to the type of lens. 


7-3. When using a C-mount Lens
[image: image41.emf]● Please assemble C-Mount adaptor.

● C-Mount adaptor is included in the package.

● Fix the lens by turning it clockwise.

7-4. When using a CS-mount Lens
[image: image42.emf]
● Take off the Protection cover. 
● Assemble the CS-MOUNT lens by turning it
  clockwise.
- When used with an Auto Iris lens, please insert
[image: image43.jpg]<MAIN MENU>

> 1. LENS MANUAL S
. EXPOSURE of
. DAY/NIGHT AUTOI
. WDR/BLC/ECLPS WDR 4
. WHITE BAL ATW
. 2D-DNR DISABLED
. 3D-DNR ONd
. EFFECTS o
. SPECIAL o
0.EXIT RETJ

<MANUAL IRIS MENU>

»BRIGHTNESS 50010
REACTION SPEED 8
SAVE




 the iris jack into the Iris socket which is located
 on the right side of the camera.
- Please set the lens selection to DC on the OSD setting.
Caution[image: image44.emf]
· Use the lens as shown in the picture here. 
· When the lens is out of the spec, it could damage the camera inside or may not be correctly fitted with the camera. 

· Use the lens less than 450g. Heavier lens can damage the camera.
7-5. Video Monitor 
Connect Video cable with BNC-male connector to the monitor

[image: image45.png]



7-6. Impedance alliance in the system connection.

Make sure to connect the cable with the power off. Please locate the impedance 

switch of each equipment as instructed. Locate the switch to Hi-z position for the interim equipment. Locate the switch to 75Ω for the last equipment.

[image: image46.emf]
7-7. Power
Please check if the model is 12VDC or 12VDC / 24VAC Dual power before the installation. Please refer to the voltage description on the rear panel to choose the right power source.
[image: image47.emf]● 12VDC camera 

Please make sure to use a UL/CE approved and 12VDC/500mA regulated power supply.

● 12VDC/24VAC Dual power camera

You can use either 12VDC Power or 24VAC Power source with non-polarity connection.

8. OSD Manual for functions control
[image: image48.jpg]LENS

EXPOSURE

DAY/NIGHT

WDR/BLC/ECLPS

WHITEBAL

2D-DNR

3D-DNR

EFFECTS

SPECIAL

EXIT

MANUAL
AGC
AUTOI
OFF
ATW
DISABLED

ON

DZOOMMODE

COLOR ADJUST

CAM NAME

MONITOR
TYPE

MAIN MENU

DC
SHUTTER SENS - UP
AUTO2 DAY

WDR BLC
CoL-ROLL PUSH

OFF Low

OFF

IMAGE D-EFFECT
FREEZE

TARGET CROSS | BAD PIXEL

MOTION DET PRIVACY
MASK

FACE REC COMM SET

INITIAL

NIGHT

ECLPS

MANUAL

MIDDLE

CONTRAST

INITIAL SET

LANGUAGE

FACTORY
RESET

SAVE

HIGH

SHARPNESS

SAVE

STABILIZER

SAVE



8-1. On Screen Menu (OSD) 

[image: image49.jpg]



8-2. Setting up the menu

1. Press the Tact SW to enter <MAIN MENU>. 

2. Move the arrow to a specific menu 

by pushing the Tact SW up or down.

3. Adjust the selected feature by pushing the Tact SW left or right

[image: image50.jpg]VIDEO

VIDEO
75Q N\ Hi~z 750(— Hi-z
D) Va ~
O O| 10 O
IN out N out
A—a A
=3




[image: image51.jpg]


       
4. When completed, move the arrow indicator to ‘EXIT’ and press the Tact SW to finish all settings.

☞ Note

[image: image52.jpg]C-MOUNT Lens : Less than 11mm
CS-MOUNT Lens : Less than 6mm



· If ‘   ‘  ‘ indication appears on a selected sub-menu, press the Tact SW to get into a following
menu.

· If ‘ --- ‘ indication appears on any sub-menu, it means the function with this mark is deactivated.

            (Ex : When SENS-UP is OFF, 3-DNR becomes deactivated ‘---‘ )  

[image: image53.jpg]



8-3. LENS  
Select the lens and adjust the SETUP suitable to the type of lens.

◆ When DC lens is used. (※ Default)
1. Please select the lens mode to DC IRIS.

2. Press the Tact SW to enter the <DC IRIS MENU>.

- BRIGHTNESS: adjust the brightness of screen when the environment is extremely bright or dark.  

- REACTION: Reaction speed of the shutter is adjustable. It is useful when the light condition of 
                 the environment keeps changing quickly. 

[image: image54.jpg]


3.  Save 
     Press SAVE to finish all settings and return to <MAIN MENU>
◆ When MANUAL lens is used.

1. Please select the lens mode to MANUAL.

2. Press the Tact SW to enter the <MANUAL IRIS MENU>.

-  BRIGHTNESS: same as above

-  REACTION: same as above

[image: image55.jpg]


8-4. EXPOSURE

(1) AGC (Auto Gain Control)

- OFF: Deactivates the AGC function.

- LOW: Allows automatic gain control from 0 to 20dB. (※ Default)
- MIDDLE: Allows automatic gain control from 0 to 30dB.

- HIGH: Allows automatic gain control from 0 to 42dB.

※ The higher the gain level, the brighter the screen becomes while the higher the noise as well.

[image: image56.jpg]Lens Cable

Connector

Pin3
Pin1

e

Pin 4
Pin 2




(2) SHUTTER

A.  AUTO : The shutter speed is controlled automatically. 
(※ Default)
B.    FLK  : Please select FLK mode when flickering occurs on the screen, due to an imbalance

between illumination and frequency.

C.  FIXED : The shutter speed is controlled manually.

   - You can select the speed from ‘1/60’ to ‘1/90,000 sec. (NTSC), ‘1/50’ to ‘1/90,000 sec. (PAL).

   - You can select Digital Slow Shutter between  x2 ~ x512. (DSS)
※ Please use FIXED mode only when the light level of the surveillance area is fixed all the time.

[image: image57.jpg]


(3) SENS-UP
- Low light sensitivity is greatly improved with the use of this function.
- Low light sensitivity level is adjustable from 2 to 512 times.

- When SEN-UP level is higher, the picture becomes brighter while noise is increased at the same
rate of SENS-UP.

ON : Adjustable SENS-UP level between  x2 ~ x512.

         OFF: SENS-UP is disabled.

※ Default setting is <x8>
☞ Note

· When the SHUTTER is set to MANUAL mode, SENS-UP does not operate.

· When the AGC is set to OFF, SENS-UP does not operate.

(4) INITIAL SET

Initialize all settings of EXPOSURE menu.

(5) SAVE

[image: image58.jpg]


8-5. DAY / NIGHT
It is used to improve the sensitivity and clarity of picture quality at low light
- DAY    :  The image is always displayed in color.

- NIGHT :  The image is always displayed in black and white.  

- AUTO1 :  The image is automatically converted to B/W from Color at 3 lux. 

 ● IR WDR LEVEL: Select  Anti- IR saturation level - OFF/LOW/MIDDLE/HIGH

● IR WDR ADJUST: Adjust the value in each of selected level - 0 ~ 6 

- AUTO2: The image is automatically converted to B/W from Color at 3 lux. 

          · D/N LEVEL: LOW/MIDDLE/HIGH

          · FILTER DLY:  Select the duration time about changing the Day/Night mode. 

                             (1 SEC, 3 SEC, 5 SEC, 10 SEC, 15 SEC, 30 SEC, 60 SEC)                                        

[image: image59.jpg]


          · NIGHT BURST: Burst ON/OFF selectable.                      

          · INITIAL SET: Initialize all settings of AUTO2 menu.

※ Default setting
    - Infrared camera: AUTO1
    - Non-Infrared camera: AUTO2
8-6. WDR/BLC/ECLPS

The image quality is greatly improved with the use of this mode when there is a strong backlight behind the object.

- OFF : Deactivates the WDR function. 

- WDR: Camera scans both of bright lit background and shaded area
distinctively under an extremely contrasted light situation. (LOW/MIDDEL/HIGH)              

- BLC: Enables a user to directly select a desired area from a picture and to view the area more
 clearly.(LOW/MIDDEL/HIGH)



-ECLIPSE: 

· Can mask the head lights to view car license number plate more clearly.
· Can select masking color.
· Adjust the ECLIPS level at the GAIN CONTROL as OFF mode.

[image: image7.jpg]<MAIN MENU>

. LENS DC IRISd
. EXPOSURE &

. DAY/NIGHT AUTOI
.WDR/BLC/ECLPS WDRJ

.WHITE BAL ATW
. 2D-DNR Low
.3D-DNR ONJ
. EFFECTS 4
. SPECIAL J
. EXIT RET 4





8-7. White Balance (WHITE BAL)

This is useful to optimize the white balalnce control under a certain artificial lighting area where a standard white balance condition is not suitable.

- ATW (Auto Tracking White Balance)

        This mode can be used within the color temperature
range of 1,500˚K~11,000˚K. (※ Default) 

- COL-ROLL

       Anti color rolling mode. 

       Use it only when the rolling is severely shown.           

- MANUAL 

Recommanded to use it when the light temperature is fixed all
 the time Select INDOOR or OUTDOOR mode.

- PUSH (Push lock)

      Use this when the White Balance is deviated. 

Face the camera toward the white wall or white paper and press the Tact SW  

 ☞ Note

White Balance may not work properly under the following condition

· When the color temperature of the environment surrounding the subject is out of the

control range.(e.g. clear sky or sunset)
· When the ambient illumination of the subject is dim.

· If the camera is directed towards a fluorescent light or is installed in a place where 

  illumination change dramatically, the White Balance operation may become unstable.
8-8. 2D-DNR (2D Noise Reduction)

- LOW

- MIDDLE

- HIGH 

- DISABLED: Deactivates 2DNR. Noise is not reduced.

      - OFF : Activates 2DNR(Level : 0). Noise is not reduced.

☞ Note

<Bad pixel detection> function is operated only when <2D-DNR DISABLED> is selected.
8-9. 3D-DNR (3D Noise Reduction)


The background noise in the low light level decreases automatically 

as the level of gain changes.

- DNR LEVEL

  Adjust the noise reduction level. 

- DESTINATION

           Adjust the 3DNR gain.         

- GLOBAL MOVEMENT

           Control ghost phenomenon

8-10. EFFECTS  
  - D-ZOOM : You can use a digital zoom.

      · ZOOM : x1 ~ x8.30

· DZOOM PAN : -100 ~ 100

       · DZOOM TILT : -100 ~ 100 

- IMAGE FREEZE : You can view still or live image.

- D-EFFECT 

· V-FLIP : You can flip the picture vertically on the screen.

       · MIRROR : You can flip the picture horizontally on the screen. 

· ROTATE : You can flip the picture rotatively on the screen.

- CONTRAST : 0 ~ 100

- SHARPNESS : 0 ~ 100

- COLOR ADJUST
· COLOR  : ON/OFF

· COLOR LEVEL : 0 ~ 200

. COLOR HUE : -180 ~ 180

. RED GAIN : -100 ~ 100

. BLUE GAIN : -100 ~ 100

- TARGET CROSS : Display ‘╇’ on the screen

     · COLOR  : BALCK,GRAY,WHITE,RED,GREEN,YELLOW

· MOVE X : -100 ~ 100

. MOVE Y: -100 ~ 100

. INITIAL SET 

. SAVE
☞ Note

At TARGET CROSS function is in operation, ‘╇’  display shall disappear if MOTION DET
function is ‘ON’

- BAD PIXEL: BAD PIXEL compensation

· BP DETECTION: Make the screen completely black by covering
 the camera.  

· SEARCH AREA: X START/Y START/WIDTH/HEIGHT

. THRESHOLD: 40 ~ 1024

. SAVE

(※ Users may simply activate BP DETECTION and go directly to SAVE. Then Bad pixels in
 full screen are compensated automatically)

- INITIAL SET : Initialize all settings of EFFECTS menu.

(※ TARGET CROSS and  BAD PIXEL modes are not initialized) 

- SAVE 

8-11. SPECIAL MENU

CAM NAME

· Please position the arrow to point to ’CAM NAME’ by using the

    UP or DOWN selection.

· Please select ‘ON’ by using the LEFT and RIGHT selection. 

Please press the button to complete ‘ON’.

☞ Note
If ‘OFF’ is selected, the CAM NAME does not appear on the monitor
even if it has been input.

· Up to 15 letters are offered for the CAM NAME.

① Please move the cursor to the letter to choose by using the UP and DOWN selections.
② Set an ID from A~Z /a~z /0~9 by using the UP, DOWN, LEFT and RIGHT Selections.

③ Lock in the letters by using the button.

 · When the letter is locked in, the cursor moves to the next space.

④ Please repeat the above to input CAM NAME.

☞ Note
· For a wrong name has been input,

- If you press the button after moving the cursor to CLR, all 
letters will be erased.

- If you want to correct a letter, please move the cursor to the arrow at the bottom left of the

screen and press the button.

- Please position the cursor on the letter you wish to correct, and then move the cursor onto

the letter you wish to choose and press the button.

· When a name has been chosen, please select a position for the name to display.


① Please move the cursor onto ’POS’ and then press the button.

② The name will appear at the top left hand corner.

③ Please find the position you wish to display the name by using the 4 directional 

 selections, and then press the Button.

· Please select ‘END’ and then press the Button to complete CAM NAME input.

MOTION (Motion Detection: MD)

- ZONE NUMBER : You can select up to 8 MD areas.                  

- ZONE STATE : Determines whether to use the MD area selected in 
ZONE NUMBER.

- WIDTH : Determines the coordinate of the horizontal size.

- HEIGHT : Determines the coordinate of the vertical size.

- MOVE X : Determines the coordinate of the horizontal axis.   

- MOVE Y : Determines the coordinate of the vertical axis.

- SENSITIVITY : Determines the coordinate of the sensitivity. 

- INITIAL SET : Initialize all setting of MOTION DET menu. 

- SAVE : Select this to save the MOTION menu setting and return to the
SPECIAL menu.

PRIVACY (Privacy Masking)

- MASK NUMBER : You can select up to 8 PRIVACY areas.

- MAKS STATE : Determines whether to use the selected in 

AREA SELECT.

- SENSITIVITY : Determines area color. You can select GRAY, 

WHITE, RED, GREEN, BLUE, YELLOW, BLACK.

- WIDTH : Determines the coordinate of the horizontal size.

- HEIGHT : Determines the coordinate of the vertical size.

- MOVE X : Determines the coordinate of the horizontal axis.   

- MOVE Y : Determines the coordinate of the vertical axis.

- INITIAL SET : Initialize all setting of PRIVACY menu.

- SAVE : Select this to save the PRIVACY menu setting and return 

to the SPECIAL menu.


LANGUAGE

You can select the menu language according to your requirements. 

- English, Korean, Russian, Spanish, French

STABILIZER  (DIS)
This function mitigates any picture movement due to external factors 

such as wind.

☞ Note
· The chance of resolution decrease is existed because  STABILIZER 

function uses the digital zoom. 

· STABILIZER doesn’t operate when background illumination is too low.

· STABILIZER doesn’t operate when object pattern is monotonic like  sky or white wall.


MONITOR TYPE

- CRT : GAMMA 0.45

- LCD : GAMMA 1.0

FACE REC (Face Recognition) 

A special area where the internal illumination is dark while strong back light exists behind the object, normally human face becomes unrecognizable. This mode is suited for this particular situation.

- FR BRIGHTNESS : 0 ~ 40

- FR CONTRAST : 0 ~ 40

- COLOR SAT : 0 ~ 200

- CONTRAST : 0 ~ 100

- OUTPUT LEVEL : -64 ~ 64

- SHARPNESS : 0 ~ 100

COMM SET (Communication seting)

This function sets up the camera communication status when controlling the camera 

through an external control device.


- CAMERA ID   

 · CAMERA ID : Determines the camera’s identification

 number (between 0 and 255).

·  ID POSITION: Displays camera title on top left & right side of
the screen.

·  BAUD RATE :  You can select 4800/9600/19200/38400/57600 bps . 

·  SAVE/REBOOT: Select this to save the COMM SET menu setting and  return to the main screen.

·  PREVIOUS: Return to COMM SET menu.

☞ Note

-  RS-485 cable : Please connect (-) firstly and then (+).

-  IF there happens RS-485 cable conection error or any other problems, a message
‘ERROR RS-485 Connection. Please Reconnect and Reboot’ is displayed. 
Please reset the power or OSD.  

FACTORY RESET

Resets the camera setting to the factory defaults.

But  <COMM SET> menu is not initialized. 

8-12. EXIT

Save all setting menus by pressing the Tact SW and then exit by pressing EXIT menu.

9. Troubleshooting
If there are problems in operating, please refer to the checklist below. If the problem persists, please contact the agent where this product is purchased.

	Problems
	Troubleshooting

	Nothing appears on the screen.
	· Please check that the power cord and line connection

between the camera and monitor are fixed properly.

· Please check that you have properly connected VIDEO

cable to the camera VIDEO output jack.

	The image on the screen is dim.
	· Is lens stained with dirt? Clean your lens with soft,

clean cloth.

· Set the monitor to proper condition.

· If the camera is exposed to too strong light, change

the camera position.

	The image on the screen is dark.
	· Adjust the contrast feature of the monitor.

· If you have an intermediate device, set the 75Ω / Hi-Z properly  

(Please check the impedance)

	The camera is not working

properly, and the surface

of the camera is hot.
	· Please check that you have properly connected the camera

to an appropriate power source.

	The DAY/NIGHT menu

does not work.
	· Please check if DAY NIGHT menu is set to ‘AUTO1’.

	The SENS-UP function  

does not work.
	· Please check that AGC of EXPOSURE SETUP menu is set to ‘OFF’.

· Please check that SHUTTER of EXPOSURE SETUP menu is set to
‘A.FLK’ or ‘MANUAL’.

	The MOTION DETECTION

function is not working.
	· Please check if MOTION DETECTION function is turned on.

· Please check if MD LEVEL is too low.

· Please check the setting of the MD area.

	Color is not correct.
	· Please check the setting of WHITE BAL SETUP menu .

	The screen flickers continually.
	· Please check that direction of camera turns toward the Sun.


	Problems
	Troubleshooting

	RS-485 

communication fails.
	· Please check the polarity between RS-485 Control Port

and RS-485 cable.



	
	485 Control Board Connection Port
	RS-485 Control Port

	
	(+)CONNECTION TERMINAL(TRX+)
	485+

	
	(-)CONNECTION TERMINAL(TRX-)
	485-

	
	· Please check if the initial value of the RS-485 Communication 
setup matches between camera and OSD Controller or DVR.
*RS-485 Communication establishment initial value

	
	Item
	Camera ID
	BAUD RATE
	UART MODE
	RET PKT

	
	Initial value
	1
	9600
	8-NONE-1
	ENABLE

	
	· We recommend that you make ground connection
between camera and controller for safe communication control.

· When RS-485 cable is not properly connected, 
A warning message ‘ERROR RS-485 Connection. Please reconnect and 

Reboot’ is displayed. 
Then please turn off the power and connect  the cable again.



10. Dimension 

[image: image8.jpg]126.0

21.0 95.0

© P | e





10. Specification

	MODEL
	VC58S

	Image sensor
	1/3” Sony Super HAD CCD II

	Effective pixels
	NTSC : 768(H) X 494(V)

PAL : 752(H) X 582(V)

	H.Resolution
	600TV Lines(Day)/650TV Lines(Night)

	Synchronizing system
	Internal

	Scanning system
	NTSC : 525 Lines   PAL : 625 Lines   2:1 Interlaced

	Video output
	1.0Vp-p Composite 75Ohms

	S/N ratio
	More than 52dB(AGC Off)

	Min.Illumination
	0.1Lux at F1.2(Sens-up Off), 0.0001Lux at F1.2(Sens-up, x512)

	Shutter speed
	NTSC : 1/60~90,000sec. PAL : 1/50~90,000sec.

	Gamma control
	Standard γ=0.45

	White Balance
	1500°K ~ 11,000°K Auto.

ATW/Cor-Roll/PUSH/MANUAL

	Gain control
	Standard : -6dB ~ 34dB Auto / Maximum by OSD

	Smear effect
	0.005%

	Power source
	12VDC, 12VDC / 24VAC

	Operating current
	200mA / 12VDC
90mA / 24VAC

	Iris control
	DC / MANUAL

	Operating temp
	-10°C ~ 50°C

	Humidity
	0%RH ~ 90%RH

	Measurement(mm)
	66(W) x 57(V) x 126.5(L)

	Weight(Approx.g)
	550

	Function

	WDR
	Off/On(User selectable by OSD)

	2D/3DNR
	Off/On(User selectable by OSD)

	Sens-up
	Off/x2 ~ x512

	Motion detection
	Off/On(8 Zone setting available)

	Digital zoom
	Off/x1 ~ x8.30

	White balance
	ATW/Cor_Roll/PUSH/MANUAL

	Privacy mask
	Off/On(8 Zone setting available)


� EMBED StaticMetafile ���





� EMBED StaticMetafile ���





Protective cover








C-Mount lens adaptor





Auto IRIS conn.








Day/Night mechanical 


filter changer


(Inside)








Intelligent


Photo sensor








1/3” Sony super HAD


CCDII & WDR chipset








Back focus adjustment Knob 


(Tolerance +� / - 0.5mm)








Fixture








Fixture can be separated and attached on the bottom side as well. 


Please use the supplied clamp screws only to change the position of the bracket.


Using screws more than 5mm depth can do a serious damage to the inside of


this camera.  


* Use an international standard 1/4”-20 mounting screw to install the camera.  








Fixture





1/4”-20 mounting thread





Power indicator








Video out


(BNC)








OSD function control key








Power In


(Non polarity)








RS-485








How to use OSD button (Tact SW)


Press 


to enter the menu


to save settings


Push up or down


to select functions 


Push left or right


to adjust a level or mode of sub-menu








       








                                                               1                                              version 1.0.1

_1384084456.wmf

_1384084457.wmf

_1384084455.wmf

