HIGH RESOLUTION

COLOR CCD CAMERA

OSD OWNER’S MANUAL

용

■ Contents

| 1. OSD Menu Structure | --------------------------------- | 3 |
| --- | --- | --- |
| 2. Setting up the Menu | --------------------------------- | 5 |
| 3. Menu Operation - CAM TITLE - LENS - EXPOSURE - WHITE BALANCE - DAY& NIGHT - BACKLIGHT - SPECIAL - IMAGE ADJ. - LANGUAGE - RESET 4. Troubleshooting | --------------------------------- --------------------------------- --------------------------------- --------------------------------- --------------------------------- --------------------------------- --------------------------------- --------------------------------- --------------------------------- --------------------------------- --------------------------------- --------------------------------- | 6 6 6 6 7 7 8 9 11 12 12 13 |

1. OSD Menu Structure

2. Setting up the Menu

1) Use five buttons on keyboard of the camera or joystick controller.

- Push UP, DOWN, LEFT, RIGHT, MENU(SET) button

- Control UP, DOWN, LEFT, RIGHT, MENU(push the joystick)

UP

DOWN

L

R

2) To open and exit the menu screen.

- Press MENU button.

The menu screen appears on the monitor.

Check the current settings on the menu.

- Select OSD item using UP, DOWN button.

- Set details using LEFT, RIGHT button.

- Select EXIT and then press the MENU button to finish the menu screen.

Note) The menu screen will be disappeared after save the current setting value.

3) To reset the camera to factory setting.

Select RESET.

Press MENU button.

4) RETURN: Return to the previous.

3. Menu Operation

SETUP

▶CAM TITLE        OFF

LENS             DC

EXPOSURE         ↙

WHITE BAL.       ATW1

DAY&NIGHT     AUTO↙

BACKLIGHT        OFF

SPECIAL            ↙

IMAGE ADJ.          ↙

LANGUAGE     ENGLISH

RESET              ↙

EXIT

CAM TITLE

Select camera title menu (Text edit).

- LENS

Select lens type

▶ DC / VIDEO / MANUAL

- EXPOSURE

SETUP

CAM TITLE        OFF

LENS             DC

▶EXPOSURE         ↙

WHITE BAL.       ATW1

DAY&NIGHT     AUTO↙

BACKLIGHT        OFF

SPECIAL            ↙

IMAGE ADJ.          ↙

LANGUAGE     ENGLISH

RESET              ↙

EXIT

EXPOSURE

▶SHUTTER         1/60

BRIGHTNESS       44

AGC             ----

DWDR           OFF

RETURN        RET↙

● SHUTTER: You can set to auto or manual mode.

▶ AUTO, 1/60(50), FLK, 1/250, 1/500, 1/1000, 1/2000, 1/4000, 1/5000, 1/10000, 1/100000sec

● BRIGHTNESS: You can adjust brightness level.

▶ 0 (dark) ~ 255 (bright) steps

● AGC: You can select auto gain control level.

▶ OFF / LOW / MIDDLE / HIGH

● DWDR: You can select WDR (Wide Dynamic Range) level.

▶ DWDR: OFF / ON

▶ LEVEL: 0 ~ 35 steps

- WHITE BALANCE

SETUP

CAM TITLE        OFF

LENS             DC

EXPOSURE         ↙

▶WHITE BAL.       ATW1

DAY&NIGHT     AUTO↙

BACKLIGHT        OFF

SPECIAL            ↙

IMAGE ADJ.          ↙

LANGUAGE     ENGLISH

RESET              ↙

EXIT

WB MANUAL

▶COLOR TEMP     MANUAL

BLUE                111

RED                 095

RETURN           RET↙

♦ Select ATW1 / ATW2 / AWC→SET / MANUAL(OUTDOOR / INDOOR / MANUAL)

● ATW1: Automatically adjusts color according to the available lighting. (2500°K → 9000°K)

● ATW2: Automatically adjusts color according to the available lighting. (2000°K → 11000°K)

● AWC→SET: It is a fixed white balance mode that may be automatically                            readjusted only by pressing MENU Button.

● MANUAL

▶ OUTDOOR: Set color temperature to be Outdoor light (6300°K.)

▶ INDOOR: Set color temperature to be Indoor light (4200°K.)

▶ MANUAL: Color can be corrected.

▷ BLUE GAIN: Adjust B gain value 0 ~ 255

▷ RED GAIN: Adjust R gain value 0 ~ 255

- DAY&NIGHT

SETUP

CAM TITLE        OFF

LENS             DC

EXPOSURE         ↙

WHITE BAL.       ATW1

▶DAY&NIGHT     AUTO↙

BACKLIGHT        OFF

SPECIAL            ↙

IMAGE ADJ.          ↙

LANGUAGE     ENGLISH

RESET              ↙

EXIT

D&N AUTO

▶DN⇒LEVEL          184

DN⇒DELAY        3 SEC

ND⇒LEVEL          018

DN⇒DELAY        3 SEC

RETURN          RET↙

- Select AUTO / COLOR / B/W / EXT

● AUTO: Day(color) & Night(B/W) Auto mode

▶ D→N LEVEL: 128 ~ 192 steps

▶ D→N DELAY: 1 ~ 30 SEC

▶ N→D LEVEL: 0 ~ 64 steps

▶ N→D DELAY: 1 ~ 30 SEC

● COLOR: always color

● B/W: always black-and-white

▶ BURST: OFF / ON

▶ IR SMART: ▷ OFF / ON

▷ IR GAIN: 0 ~ 255

▷ HEIGHT: 0 ~ 15

▷ WIDTH: 0 ~ 15

▷ LEFT/RIGHT: 0 ~ 15

▷ TOP/BOTTOM: 0 ~ 15

▶ IR LEVEL: HIGH / LOW

● EXT: Switches between color and black-and-white picture

when an external switching signal is received.

BLC

▶AREA SEL         AREA1

AREA STATE          ON

GAIN                042

HEIGHT              005

WIDTH               004

LEFT/RIGHT           006

TOP/BOTTOM          005

RETURN          RET↙

BACKLIGHT

SETUP

CAM TITLE        OFF

LENS             DC

EXPOSURE         ↙

WHITE BAL.       ATW1

DAY&NIGHT     AUTO↙

▶BACKLIGHT        OFF

SPECIAL            ↙

IMAGE ADJ.          ↙

LANGUAGE     ENGLISH

RESET              ↙

EXIT

- BACKLIGHT: Select OFF / BLC / HLC

● BLC (Back Light Compensation)

▶ AREA SEL: Select AREA1, AREA2

▶ AREA STATE: Select OFF / ON

▶ GAIN: Select 0 ~ 255 steps

▶ HEIGHT: Select 0 ~ 15 steps

▶ WIDTH: Select 0 ~ 15 steps

▶ LEFT/RIGHT: Select 0 ~ 15 steps

▶ TOP/BOTTOM: Select 0 ~ 15 steps

HLC

▶LEVEL              200

MODE          ALL DAY

RETURN          RET↙

● HLC (High Light Compensation)

▶ LEVEL: Select 0 ~ 255 steps

▶ MODE: ALL DAY,

NIGHT ONLY

SPECIAL

▶PARK. LINE         OFF

MOTION           OFF

PRIVACY           OFF

DPC                ↙

COMM ADJ.           ↙

VERSION         11 01 11

RETURN          RET↙

SPECIAL

SETUP

CAM TITLE        OFF

LENS             DC

EXPOSURE         ↙

WHITE BAL.       ATW1

DAY&NIGHT     AUTO↙

BACKLIGHT        OFF

▶SPECIAL            ↙

IMAGE ADJ.          ↙

LANGUAGE     ENGLISH

RESET              ↙

EXIT

● PARK.LINE: You can select parking line on the screen.

▶ LT: You can adjust the top left area on screen.(0 ~ 255 steps)

▶ LB: You can adjust the bottom left area on screen.(0 ~ 255 steps)

▶ RT: You can adjust the top right area on screen.(0 ~ 255 steps)

▶ RB: You can adjust the bottom right area on screen.(0 ~ 255 steps)

▶ F: You can adjust the front area on screen.(0 ~ 255 steps)

▶ N: You can adjust the near area on screen.(0 ~ 255 steps)

▶ T: You can adjust the parking line thickness.(0 ~ 15 steps)

● MOTION: You can detect a moving objects on the screen.

▶ AREA SEL: Select AREA1, AREA2, AREA3, AREA4

▶ AREA STATE: Select OFF / ON

▶ HEIGHT: Select 0 ~ 15 steps

▶ WIDTH: Select 0 ~ 15 steps

▶ LEFT/RIGHT: Select 0 ~ 15 steps

▶ TOP/BOTTOM: Select 0 ~ 15 steps

▶ DEGREE: Select 0 ~ 255 steps

▶ VIEW: OFF / ON

● PRIVACY: You can select Hide an area on the screen.

▶ AREA SEL: Select AREA1, AREA2, AREA3, AREA4, AREA5, AREA6, AREA7, AREA8

▶ AREA STATE: Select OFF / ON

▶ HEIGHT: Select 0 ~ 124 steps

▶ WIDTH: Select 0 ~ 200 steps

▶ LEFT/RIGHT: Select 0 ~ 200 steps

▶ TOP/BOTTOM: Select 0 ~ 124 steps

▶ COLOR: Select 0 ~ 15 steps

● DPC: Select dead pixel compensation processing

COVER THE LENS

THEN

PRESS ENTER KEY

SPECIAL

PARK. LINE         OFF

MOTION           OFF

PRIVACY           OFF

▶DPC                ↙

COMM ADJ.           ↙

VERSION         11 01 11

RETURN          RET↙

● COMM ADJ. : You can adjust command

SPECIAL

PARK. LINE         OFF

MOTION           OFF

PRIVACY           OFF

DPC                ↙

▶COMM ADJ.           ↙

VERSION         11 01 11

RETURN          RET↙

COMM ADJ.

▶CAM ID             001

BAUD RATE         9600

PROTOCOL      PELCO-D

DISPLAY ID          OFF

ID POS               ↙

RETURN          RET↙

▶ CAM ID: Select the camera ID

▷ 0 ~ 255

▶ BAUD RATE: Select serial communication speed.

▷ 2400 / 4800 / 9600 / 19200 / 38400 / 57600 bps

▶ PROTOCOL: Select operating protocol

▷ PELCO-D / PELCO-P

▶ DISPLAY ID: Select display item.

▷ OFF/ON

▶ ID POS: You can adjust ID position on screen.

● VERSION: Factory version

- IMAGE ADJ.

SETUP

CAM TITLE        OFF

LENS             DC

EXPOSURE         ↙

WHITE BAL.       ATW1

DAY&NIGHT     AUTO↙

BACKLIGHT        OFF

SPECIAL            ↙

▶IMAGE ADJ.          ↙

LANGUAGE     ENGLISH

RESET              ↙

EXIT

IMAGE ADJ.

▶LENS SHAD.         OFF

2DNR               ON

MIRROR            OFF

FONT COLOR         003

ID&TITLE             003

CONTRAST           138

SHARPNESS          021

DISPLAY           CRT↙

NEG. IMAGE          OFF

RETURN           RET↙

● LENS SHAD: You can adjust lens shading on screen.

▶ LEVEL: 0 ~ 255 steps

● 2DNR: You can select Digital Noise Reduction on screen. (OFF/ON)

● MIRROR: You can select mirror on screen. (OFF/ON)

● FONT COLOR: You can select font color on screen.

▶ FONT COLOR: O ~ 15 steps

● ID&TITLE: You can select font color on screen.

▶ ID & TITLE: O ~ 15 steps

● CONTRAST: You can adjust contrast on screen. (0 ~ 255 steps)

● SHARPNESS: You can adjust sharpness on screen. (0 ~ 31 steps)

● DISPLAY: You can adjust display. (CRT / LCD / USER)

▶ CRT (Cathode Ray Tube, Braun tube)

▷ PED LEVEL: 0 ~ 63 steps

▷ COLOR GAIN: 0 ~ 255 steps

▶ LCD (Liquid Crystal Display)

▷ GAMMA: 0.05 ~ 1.00 steps

▷ PED LEVEL: 0 ~ 63 steps

▷ COLOR GAIN: 0 ~ 255 steps

▶ USER

▷ GAMMA: 0.05 ~ 1.00 steps

▷ PED LEVEL: 0 ~ 63 steps

▷ COLOR GAIN: 0 ~ 255 steps

● NEG.IMAGE: You can select negative display. (OFF/ON)

- LANGUAGE

SETUP

CAM TITLE        OFF

LENS             DC

EXPOSURE         ↙

WHITE BAL.       ATW1

DAY&NIGHT     AUTO↙

BACKLIGHT        OFF

SPECIAL            ↙

IMAGE ADJ.          ↙

▶LANGUAGE     ENGLISH

RESET              ↙

EXIT

● ENGLISH / CHINESE

- RESET

Factory default reset.

SETUP

CAM TITLE        OFF

LENS             DC

EXPOSURE         ↙

WHITE BAL.       ATW1

DAY&NIGHT     AUTO↙

BACKLIGHT        OFF

SPECIAL            ↙

IMAGE ADJ.          ↙

LANGUAGE     ENGLISH

▶RESET              ↙

EXIT

4. Troubleshooting

If there are problems in operation, please refer to the items below. If the problem persists, please contact the agent you purchased this product from.

| Problems | Troubleshooting |
| --- | --- |
| Nothing appears on the screen. | • Please check the power connection. • Please check the video signal line connection. |
| The video image is not clear. | • Please check if the lens is clean. Please clean the lens with a clean cloth or brush. • Please adjust the contrast feature of the monitor. • Please readjust the back focus of the camera. |
| The screen is dark. | • Please adjust the contrast feature of the monitor. • If you have an intermediate device, set the 75 / Hi-z properly, and check the terminals. • Please check if an auto iris lens is being used and adjust the brightness level. |
| There is a problem with the camera operation. | • Please check if an appropriate power source to the camera complies with the manufacturer's standard requirement, or if the voltage keeps changing. |
| Color is not quite right. | • Please check the 'WHITE BAL' setting |
| The screen is flickering. | • Please check if the camera is facing directly into sunlight or fluorescent light. • Please check if an auto iris lens is being used. • Please check the connection of the lens connector cable. |
| 2DNR and DAY&NIGHT function is not working. | • Please check if the AGC menu is set to the OFF position. |

- MEMO  -

5080V-0031A