ALL IN ONE

High Speed Dome

User’s Manual

(MODEL : VPD330WD-O, VPD370WD-O)

33x, 37x Crisp Image Detection

Please read this manual carefully before installation and operation of the product.

Contents

- Features ------------------------------------------------------------------------------- 3

2.  Precaution --------------------------------------------------------------------------- 4

3.  Part Names and Functions -------------------------------------------------- 5

◆ All-in-One Outdoor PTZ Camera

◆ Package Contents

◆ DIP Switch setting

◆ Camera ID setting

◆ RS-485 Communication setting

4.  Installation ---------------------------------------------------------------------------- 13

5.  Functional description ---------------------------------------------------------- 32

◆ Setup Menu Overview

◆ Main Menu

1) PAN/TILT setting

2) Camera setting

3) Autoseq setting

4) Zone setting

5) Alarm setting

6) Initialize setting

6.  Product Structure ---------------------------------------------------------------- 32

7.  Specifications ---------------------------------------------------------------------- 33

- Features

* Dynamic and precise AF control

The camera can trace and pick up the object, moving fast from the scene with powerful 324 times(Optical 27x, Digital 12x), 396 times(Optical 33x, Digital 12x), 444 times(Optical 37x, Digital 12x) zoom by greatly improved Auto Focus control.

* Removable Motorized IR Cut Filter(ICR)

* Viewable at total darkness

The image at daytime displays as True Color with IR Cut filter attached and turns to

Clear B/W(color signal removed) mode at night(True Day/Night). With ICR, it comes to IR sensitive and enables the camera to view the scene at total darkness.

*Super Wide Dynamic(sWDR)/ Only VPD330WD,VPD370WD

With new powerful DSP system integrated, it makes more 126 times upgraded Backlight

compensation than that of existing BLC system

* High resolution of 550TVL(Color)/ 680TVL(Night)

* Powerful 27x/ 33x/ 37x optical zoom

* Extremely upgraded sensitivity of 0.7Lux(Sens-up off)/ 0.002Lux(Sens-up, 256x)

/ 0.06Lux(B/W)

Clear difference with built –in Super DSP system and ICR mechanism is to surveil Objects clearly at even a starless dark night. In addition the built –in Dynamic Noise Reduction(DNR) removes noises and ghost effect to have the picture quality greatly improved.

It also saves the DVR’s recoding capacity up to 70%.

* Quick and Easy installation

Dome base is separated and easily mounted on the ceiling and the main body is simply hooked on it by hand with no tool. No complicated cable connection is additionally required with the built-in one-stop connection mechanism

- Precaution

Safety Precautions

* The purpose of this information is to ensure proper use of this product to prevent danger to damage to property. Please be sure to observe all precautions

* The precautions are divided into “Warnings” and “Cautions” as distinguished below;

Warning : Ignoring these warnings may result in death or serious injury.

Caution : Ignoring these cautions may result in injury or damage to property.

| Warning :instructions alert you to the potential risk of death or serious injury | Caution : instructions alert you to the potential risk of injury or damage to property |
| --- | --- |

Warnings

1. Be sure to use only the standard adapter which is specified in the specification sheet.

Using any other adapters could cause fire, electrical shock, or damage to the product.

2. When connecting the power supply and signal wires, check the external connection

terminal before connecting them. Connect the alarm signal wires to the alarm terminals,

the AC adaptor to the AC power input receptacle,

and the DC adapter to the DC power Input, making sure that the correct polarity is observed.

(Connecting the power supply incorrectly may cause fire, electrical shock,

or damage to the product.)

3. Do not connect multiple cameras to a single adapter.

(Exceeding the capacity may cause abnormal heat generation or fire.)

4. Securely plug the power cord into the power receptacle.

(Insecure connection may cause fire.)

5. When installing the camera on a wall or ceiling, fasten it securely and firmly.

(A falling camera may cause personal injury)

6. Do not place conductive objects(e.g. screwdrivers, coins, and metal things) or containers filled with water on top of the camera.

(Doing so may cause personal injury due to fire, electrical shock or falling objects)

7. Do not install the unit in humid, dirty, or sooty locations

(Doing so may cause fire or electrical shock.)

8. If any unusual smell or smoke comes from the unit, stop using the product.

In such case, immediately disconnect the power source and contact the store of purchase.

(Continued or modify this product in any way.)

9. When clean in use in such a condition may cause fire or electrical shock.)

10. If this product fails to operate normally, contact the store of purchase.

Never disassemble g, do not spray water directly onto parts of the product.

(Doing so may cause fire or electrical shock.)

Wipe the surface with a dry cloth. Never use detergents or chemical cleaners on the product, as this may result in discoloration of surface or cause damage to the finish.

Cautions

- Do not drop objects on the product or apply strong shock to it. Keep away from a location subject to excessive vibration or magnetic interference.

- Do not install in a location subject to high temperature(over 122℉), low temperature

(below 14℉) or high humidity. (Doing so may cause fire or electrical shock.)

- Avoid a location which is exposed to direct sunlight or near heat sources such as heater or radiators. (Neglecting to do so may result in a risk of fire.)

4. If you want to relocate the already installed product, be sure to turn off the power and then

move or reinstall it.

5. Install in a well-ventilated location.

6. Remove the power plug from the outlet when there is a lightning storm.

(Neglecting to do so may cause fire or damage to the product.)

3. Part Names and Function

◆ All-in-One Outdoor PTZ Camera

Video output

Power (24VAC)

RS-485 DATA

Relay output

Alarm Input

POWER Cable

| COLOR | DESCRIPTION |
| --- | --- |
| YELLOW | EARTH |
| BROWN | RS485A |
| WHITE | RS485B |
| BLUE | AC24V+(Temperature Control Power) |
| Green | AC24V-(Temperature Control Power) |
| RED | AC24V+(Mechanism Power) |
| BLACK | AC24V-(Mechanism Power) |

Alarm In/out Cable

| COLOR | DESCRIPTION |
| --- | --- |
| BLACK | GND |
| YELLOW | ALARM INPUT 1 |
| Green | ALARM INPUT 2 |
| RED | ALARM INPUT 3 |
| WHITE | ALARM OUT |
| BLUE | COM |

◆ Package Contents

Instruction manual

1EA

Rubber Ring

1EA

Wide Dynamic 27x, 33x, 37x

Zoom Speed Dome Camera 1EA

[ Option ]

Adaptor(AC24V/2A)

1EA

Adaptor power plug

1EA

Tapping screw

4EA

Plastic anchor

4EA

Wrench

1EA

Fixed screw

4EA

z Note : Mount Brackets are optional

| ● Wall Mount Bracket | ● Ceiling Mount Bracket |
| --- | --- |
| ● Wall Mount with Junction Box | ● Corner and Pole Mount Bracket |

◆ DIP SWITCH SETTINGS

In a configuration where the camera’s RS485 data port is used for Camera control(pan, tilt, etc.) by the system controller, the camera’s DIP switches must be configured to specify the camera ID and communication parameters.

The camera mounting base needs to be removed to access the DIP switches.

See step 1 & 2 on page 11 for the information about how to remove the camera mounting base.

◆ Camera ID Setting

The factory default settings of these DIP switches are ID 1

(1-ON,0-OFF) SW1~SW8

| DIP SW | ID | DIP SW | ID | DIP SW | ID |
| --- | --- | --- | --- | --- | --- |
| 10000000 | 1 | 10101000 | 21 | 10010100 | 41 |
| 01000000 | 2 | 01101000 | 22 | 01010100 | 42 |
| 11000000 | 3 | 11101000 | 23 | 11010100 | 43 |
| 00100000 | 4 | 00011000 | 24 | 00110100 | 44 |
| 10100000 | 5 | 10011000 | 25 | 10110100 | 45 |
| 01100000 | 6 | 01011000 | 26 | 01110100 | 46 |
| 11100000 | 7 | 11011000 | 27 | 11110100 | 47 |
| 00010000 | 8 | 00111000 | 28 | 00001100 | 48 |
| 10010000 | 9 | 10111000 | 29 | 10001100 | 49 |
| 01010000 | 10 | 01111000 | 30 | 01001100 | 50 |
| 11010000 | 11 | 11111000 | 31 | 11001100 | 51 |
| 00110000 | 12 | 00000100 | 32 | 00101100 | 52 |
| 10110000 | 13 | 10000100 | 33 | 10101100 | 53 |
| 01110000 | 14 | 01000100 | 34 | 01101100 | 54 |
| 11110000 | 15 | 11000100 | 35 | 11101100 | 55 |
| 00001000 | 16 | 00100100 | 36 | 00011100 | 56 |
| 10001000 | 17 | 10100100 | 37 | 10011100 | 57 |
| 01001000 | 18 | 01100100 | 38 | 01011100 | 58 |
| 11001000 | 19 | 11100100 | 39 | 11011100 | 59 |
| 00101000 | 20 | 00010100 | 40 | 00111100 | 60 |
| DIP SW | ID | DIP SW | ID | DIP SW | ID |
| 10111100 | 61 | 11000110 | 99 | 10010001 | 137 |
| 01111100 | 62 | 00100110 | 100 | 01010001 | 138 |
| 11111100 | 63 | 10100110 | 101 | 11010001 | 139 |
| 00000010 | 64 | 01100110 | 102 | 00110001 | 140 |
| 10000010 | 65 | 11100110 | 103 | 10110001 | 141 |
| 01000010 | 66 | 00010110 | 104 | 01110001 | 142 |
| 11000010 | 67 | 10010110 | 105 | 11110001 | 143 |
| 00100010 | 68 | 01010110 | 106 | 00001001 | 144 |
| 10100010 | 69 | 11010110 | 107 | 10001001 | 145 |
| 01100010 | 70 | 00110110 | 108 | 01001001 | 146 |
| 11100010 | 71 | 10110110 | 109 | 11001001 | 147 |
| 00010010 | 72 | 01110110 | 110 | 00101001 | 148 |
| 10010010 | 73 | 11110110 | 111 | 10101001 | 149 |
| 01010010 | 74 | 00001110 | 112 | 01101001 | 150 |
| 11010010 | 75 | 10001110 | 113 | 11101001 | 151 |
| 00110010 | 76 | 01001110 | 114 | 00011001 | 152 |
| 10110010 | 77 | 11001110 | 115 | 10011001 | 153 |
| 01110010 | 78 | 00101110 | 116 | 01011001 | 154 |
| 11110010 | 79 | 10101110 | 117 | 11011001 | 155 |
| 00001010 | 80 | 01101110 | 118 | 00111001 | 156 |
| 10001010 | 81 | 11101110 | 119 | 10111001 | 157 |
| 01001010 | 82 | 00011110 | 120 | 01111001 | 158 |
| 11001010 | 83 | 10011110 | 121 | 11111001 | 159 |
| 00101010 | 84 | 01011110 | 122 | 00000101 | 160 |
| 10101010 | 85 | 11011110 | 123 | 10000101 | 161 |
| 01101010 | 86 | 00111110 | 124 | 01000101 | 162 |
| 11101010 | 87 | 10111110 | 125 | 11000101 | 163 |
| 00011010 | 88 | 01111110 | 126 | 00100101 | 164 |
| 10011010 | 89 | 11111110 | 127 | 10100101 | 165 |
| 01011010 | 90 | 00000001 | 128 | 01100101 | 166 |
| 11011010 | 91 | 10000001 | 129 | 11100101 | 167 |
| 00111010 | 92 | 01000001 | 130 | 00010101 | 168 |
| 10111010 | 93 | 11000001 | 131 | 10010101 | 169 |
| 01111010 | 94 | 00100001 | 132 | 01010101 | 170 |
| 11111010 | 95 | 10100001 | 133 | 11010101 | 171 |
| 00000110 | 96 | 01100001 | 134 | 00110101 | 172 |
| 10000110 | 97 | 11100001 | 135 | 10110101 | 173 |
| 01000110 | 98 | 00010001 | 136 | 01110101 | 174 |
| DIP SW | ID | DIP SW | ID | DIP SW | ID |
| 11110101 | 175 | 01010011 | 202 | 10100111 | 229 |
| 00001101 | 176 | 11010011 | 203 | 01100111 | 230 |
| 10001101 | 177 | 00110011 | 204 | 11100111 | 231 |
| 01001101 | 178 | 10110011 | 205 | 00010111 | 232 |
| 11001101 | 179 | 01110011 | 206 | 10010111 | 233 |
| 00101101 | 180 | 11110011 | 207 | 01010111 | 234 |
| 10101101 | 181 | 00001011 | 208 | 11010111 | 235 |
| 01101101 | 182 | 10001011 | 209 | 00110111 | 236 |
| 11101101 | 183 | 01001011 | 210 | 10110111 | 237 |
| 00011101 | 184 | 11001011 | 211 | 01110111 | 238 |
| 10011101 | 185 | 00101011 | 212 | 11110111 | 239 |
| 01011101 | 186 | 10101011 | 213 | 00001111 | 240 |
| 11011101 | 187 | 01101011 | 214 | 10001111 | 241 |
| 00111101 | 188 | 11101011 | 215 | 01001111 | 242 |
| 10111101 | 189 | 00011011 | 216 | 11001111 | 243 |
| 01111101 | 190 | 10011011 | 217 | 00101111 | 244 |
| 11111101 | 191 | 01011011 | 218 | 10101111 | 245 |
| 00000011 | 192 | 11011011 | 219 | 01101111 | 246 |
| 10000011 | 193 | 00111011 | 220 | 11101111 | 247 |
| 01000011 | 194 | 10111011 | 221 | 00011111 | 248 |
| 11000011 | 195 | 01111011 | 222 | 10011111 | 249 |
| 00100011 | 196 | 11111011 | 223 | 01011111 | 250 |
| 10100011 | 197 | 00000111 | 224 | 11011111 | 251 |
| 01100011 | 198 | 10000111 | 225 | 00111111 | 252 |
| 11100011 | 199 | 01000111 | 226 | 10111111 | 253 |
| 00010011 | 200 | 11000111 | 227 | 01111111 | 254 |
| 10010011 | 201 | 00100111 | 228 | 11111111 | 255 |

◆ RS485 Communication setting

Configuring DIP Switch as shown below resets communication parameters from the factory default setting by using SW9 and SW10

| SW10 | SW9 | BUAD RATE | PROTOCAL |
| --- | --- | --- | --- |
| 0 | 0 | 9600BPS | PELCO-D |
| 0 | 1 | 4800BPS | PELCO-D |
| 1 | 0 | 9600BPS | PELCO-P |
| 1 | 1 | 4800BPS | PELCO-P |

- Factory Default : 9600bps, PELCO-D

4. Installation

■ PTZ Camera with Bracket

Installation using Wall Mount Bracket

① Make a hole whose diameter is 30~40mm on the mounting surface to pass the wire(s) and cable(s) through the mounting surface. (In case of the wiring and cabling through the mounting surface only) Then prepare the wall mount bracket. Pull the wire(s) and cable(s) for the system as below. Attach the wall mount bracket to the mounting surface.

(Tapping Screw 6×50)

② When the reservation line for the wall outside connection, have to puncture

The bracket line hole first, then reserve line traversing the line hole and

The bracket hole, then fix with 4 hexagonal screws in the install position

When the cables are on the wall surface, puncture this area for cable outlet

③ High speed dome safety rope lock catch with the bracket reservation hole,

Then keeps the high speed dome a line and the corresponding electric cable

Group a line connection, picks up the high speed dome, screw tight three

Clamping screws

- Screw the dome cover to the main body and remove the protection vinyl from the dome cover.

Notice

Before starting the installation, make sure that the Camera ID and Protocol are set up properly

Installation using Wall Mount Junction Box Bracket

① Install the 4 install holes to aim at 4 expansion of metal bolt, twist the upper nut Fixedly,

loosen the water joint, reserve the electric cable group to going beyond a Line traversing

water joint then screw tight again, installation finished.

② High speed dome safety rope lock catch with the bracket reservation hole, Then keeps

the high speed dome a line and the corresponding electric cable Group a line connection,

picks up the high speed dome, screw tight three Clamping screws

③ Screw the dome cover to the main body and remove the protection vinyl from

the dome cover.

Notice

Before starting the installation, make sure that the Camera ID and Protocol are set up properly

Installation using Ceiling Mount Bracket

① When wires go though wall, cable assembly backup wires directly cross bracket hole.

Four installation holes aligns four inflation bolt and then twist nut to fasten

(Tapping Screw 6×50)

② When wires pass outside wall, firstly drill the terminal hole, and then cables

Assembly backup wires cross it and the bracket hole by turns.

Finally four installation holes aligns four installation holes aligns four inflation bolt and then

twist nut to fasten

When the cables are on the ceiling surface, first puncture this area.

③ High speed dome safety rope lock catch with the bracket reservation hole,

Then keeps the high speed dome a line and the corresponding electric cable Group a line

connection, picks up the high speed dome, screw tight three Clamping screws

- Screw the dome cover to the main body and remove the protection vinyl from the dome cover.

Notice

Before starting the installation, make sure that the Camera ID and Protocol are set up properly

OPTIONAL ACCESSORIES FOR INSTALLATION

- If installing the wall mount on an at least 80mm-long cylinder

● Pole Mount

② If installing the wall mount on a corner of the wall

● Corner Mount

5. Functional Description

◆ Setup Menu Overview

In this chapter, we will look over the overall structure of the Setup Menu and then look at the functions of each menu

PAN/TILT      ID DISPLAY     → ON/OFF

SET           CAMERA NAME → OFF/(ON)

FIRMWARE      → XXX

MANUAL SPEED → 10~110°/Sec

PROPO. P/T     → ON/OFF

DIGITAL FLIP    → ON/OFF

IMAGE HOLD   → OFF/ON

AUTORUN TIME → OFF/20~60Sec

EXIT

CAMERA

SET      BACKLIGHT    <WDR>

OFF/<WDR>/<USER> <HLC>

WDR     WDR LIMIT → LOW/MID/HIGH

WDR LEVEL → 0~100

BLC     LEVEL → LOW/HIGH

WINDOW TOP → 1~100

WINDOW BOTTOM → 1~100

WINDOW LEFT → 1~100

WINDOW RIGHT → 1~100

HLC     HLC LEVEL → LOW/HIGH

MASK COLOR → 0~10

MD      MD NUM → 1~8

MD DEFINE → OFF/ON

MD DISPLAY → OFF/ON

SENSITIVITY → LOW/HIGH

AREA TOP → 1~26

AREA BOTTOM → 5~39

AREA LEFT → 1~44

AREA RIGHT → 5~48

EXIT

ATW     ATW MODE→ ATW/MANUAL

/<HOLD>/IN DOOR

/OUT DOOR

MANUAL RED → 0~200

MANUAL BLUE → 0~200

EXIT

FOCUS     FOCUS MODE→

/ZOOM      MANUAL/AUTO/ZOOMTG

ZOOM TRK MODE→OFF/ON

ZOOM TRK SPD→FAST/SLOW

DIGITAL ZOOM→OFF/ON

ZOOM MAG→

2X/4X/6X/8X/10X/12X

ZOOM SPEED → 1~8

EXIT

AE      BRIGHTNESS → 0~100

IRIS → AUTO/MANUAL

IRIS MANUAL → F1.6 ~CLOSE

SHUTTER → ESC/MAN/A.FLK

SHUTTER MANUAL→X256~1/120000

AGC → OFF/LOW/MID/HIGH

SSNR → OFF/LOW/MID/HIGH

DSS → X2~X256, OFF

EXIT

DAY      D/N MODE → COLOR/BW/AUTO

& NIGHT      BUST MODE → OFF/ON

BURST LEVEL → 0~100

DURATION → FAST/SLOW

FILTER DELAY → 5/7/10/20/30/

40/60SEC

EXIT

SPECIAL      H-REV → OFF/ON

V-REV → OFF/ON

Stabilizer → OFF/ON

SHARPNESS → 0~100

COLOR → 0~100

EXIT

AUTOSEQ   PRESET    PRESET NO → 1~128

SET                     PRESET

DEFINE → OFF/<ON>

PRESET

NAME → OFF/<ON>

P/T

POSITION→P/T ANGLE

BRIGHTNESS → 0~100

ATW → <SET>

BLC → <SET>

EXIT

TOUR     TOUR NO → 1~5

TOUR NO DEF

→ OFF/<ON>

NAME → TOURGROUP1

TOUR NAME

DEF→OFF/<ON>

DWELL TIME → 1~120

EXIT

SCAN     SCAN NO →1~5

SCAN DEFINE → OFF/ON

NAME → SCANGROUP1

SCAN NAME

DEFINE→OFF/<ON>

PAN START POS→<180>

PAN END POS→<350>

TILT POS →<45>

SCAN SPEED

→2˚,5˚,10˚,20˚/S

EXIT

PATTERN     PATT NUMBER -> 1~2

PATT DEFINE

→ OFF/ON

NAME → PATTERN001

PATT NAME

DEF→OFF/<ON>

PATT RECORD

→OFF/<ON>

PATT SPEED

→2˚,5˚,10˚,20˚/S

EXIT

AUTO PAN     TILT ANGLE → (xxx)

PAN SPEED

→2˚,5˚,10˚,20˚/S

DIRECTION

→ CW/CCW

EXIT

AUTO MODE →

OFF/SEQ/TOUR/SCAN/PATT/A.PAN

HOME POSITION → OFF/1~128

EXIT

ZONE     AREA SEL → 1~8

SET       AREA DEFINE → OFF/ON

AREA NEW SET → OFF/ON

TOP/BOTTOM-1 → XXX

TOP/BOTTOM-2 → XXX

LEFT/RIGHT-1 → XXX

LEFT/RIGHT-2 → XXX

EXIT

ALARM     ALARM DISPLAY → OFF/ON

SET        ALARM IN → <SET>

ALARM OUT → OFF/ON

TIME OUT → 1~10SEC

EXIT

INITIALIZE      POWER ON RESET

SET            PAN/TILT INIT

CAMERA INIT

AUTOSEQ INIT

PRIVACY ZONE INIT

FACTORY INIT

EXIT

◆ Main Menu

SYSTEM INFO

This diagram shown above illustrates the

overall structure of the setup menu.

In this section, a description of the setup menu

features will enable users to tailor it to their

personal needs

MAIN MENU

1. PAN TILT SET

2. CAMERA SET

3. AUTOSEQ SET

4. ZONE SET

5. ALARM SET

6. INITIALIZE SET

7. EXIT

Selection  : UP,DOWN KEY

Menu open : IRIS OPEN Or 95+GOPRESET

Menu close : IRIS CLOSE KEY

1. PAN TILT SET

| 1.ID DISPLAY: | <ON>/OFF |
| --- | --- |
| 2. CAM NAME: | OFF/<ON> |
| 3. FIRMWARE: | XXX |
| 4. MANUAL SPEED | 10~110°/SEC |
| 5. PROPO. P/T: | ON/OFF |
| 6. DIGITAL FLIP: | ON/ OFF |
| 7. IMAGE HOLD | OFF/ON |
| 8.AUTORUN TIME: | OFF/20~60SEC |
| 9.EXIT |  |

Selection  : UP, DOWN, LEFT, RIGHT KEY

Menu open : IRIS OPEN KEY

Menu close : IRIS CLOSE KEY

■ ID DISPLAY – Camera ID appears on the screen.

OFF  : The ID does not appear on the screen.

<ON> : The ID appears on the screen.

When changing the location of the ID, press

IRIS OPEN KEY in the condition of <ON>.

Then, you can move to ID DISPLAY Menu.

■ CAM NAME – Maximum of 10 characters can be

input. When changing the camera

name, press the IRIS OPEN Key

after NAME DISPLAY is set to <ON>.

■ MANUAL SPEED

Adjustable maximum speed when PAN/ TILT

are manually controlled.

■ PROPO. P/T

Automatically adjusts the PAN/TILT movement speed according to the Zoom magnification level.

OFF: PAN/TILT movement is not automatically controlled.

ON : The PAN/ TILT speed slows down on higher

zoom magnification.

■ DIGITAL FLIP

OFF : It operates TILT below 90°.

ON  : It operates TILT over 90°.

■ FREEZE

When you move to Preset, the screen displays

the frozen image of the current displayed preset.

OFF : It displays the current image.

ON  : It displays the frozen image.

<Caution>

FREEZE works at TOUR in AUTORUN MODE.

■ AUTORUN TIME

If a PAN/TILT operation overruns its setting value AUTO REFRESH automatically compensates.

1.1. ID DISPLAY

ID location can be changed as follows.

1. Select <ON> from ID DISPLAY.

2. Press IRIS OPEN KEY.

MOVE: U/D/L/R

EXIT: IRISCLOSE

Selection : UP, DOWN, LEFT, RIGHT KEY

Menu open : IRIS OPEN KEY

Menu close : IRIS CLOSE KEY

1.2 CAMERA NAME DISPLAY

CAMERA NAME: MAINCAMERA

0123456789ABCDEFGH

IJKLMNOPQRSTUVWXYZ

DELETE : <IRISOPEN>

POSITION : <IRISOPEN>

EXIT

Selection : UP, DOWN, LEFT, RIGHT KEY

Menu open : IRIS OPEN KEY

Menu close : IRIS CLOSE KEY

The method of Camera naming is as follows;

- Select DELETE and cancel MAINCAMERA

by pressing the IRIS OPEN KEY.

- Move to the desired characters from 0 to

Z and press the IRIS OPEN KEY.

3. Repeat step 2 until the name is completed.

4. If you want to change the location of the

camera name, select position and press

the IRIS OPEN KEY.

How to change the location of camera

name is the same as changing ID DISPLAY

2. CAMERA SET

1.BACKLIGHT-----OFF/WDR/USER/HLC

2. MOTION DET --------<SET>

3. ATW --------------<SET>

4. FOCUS/ZOOM ---------<SET>

5. AE ------------------<SET>

6. DAY&NIGHT ----------<SET>

7. SPECIAL -------------<SET>

8. EXIT

2.1. BACK LIGHT

WDR(for Optical 33x/ 37x zoom only), USER

AREA & HLC are selectable.

* WDR *

It is an ideal set up for the most of different

levels of backlit or contrast lighting situations

and provides no darker subjects when there

are a dark portion and a bright portion on the

screen simultaneously.

* HLC *

It cuts off the strong light like “headlight” to

recognize a license plate or subject clear in

the entrance of parking garage or at gas station.

It doesn’t work at daytime.

* USER AREA*

User can set area to recognize the specific area

clear.

.

2.1.1. WDR LEVEL SETUP

| 1. WDR LIMIT: | LOW/MID/HIGH |
| --- | --- |
| 2. WDR LEVEL: | 0~100 |
| 3. EXIT |  |

■ WDR LIMIT

Sensitivity of WDR can be controllable to

LOW/MIG/HIGH. If the sensitivity gets higher,

the contract between dark areas and bright

areas becomes lessened.

■ WDR LEVEL

Brightness of WDR image can be wholly

adjustable.

<Caution>

1. If SHUTTER is set to MANUAL,

WDR function doesn’t work.

2. In case that color of image goes wrong

according to the lighting condition and

noise occurs in the bright area under

WDR selection, do not use WDR function.

2.1.2. BLC USER SETUP

| 1. WINDOW | OFF/ON |
| --- | --- |
| 2. LEVEL | LOW/HIGH |
| 3. WINDOW TOP | 1~100 |
| 4. WINDOW BOTTOM | 1~100 |
| 5. WINDOW LEFT | 1~100 |
| 6. WINDOW RIGHT | 1~100 |
| 7. EXIT |  |

Above menu is selectable and adjustable by

controller.

2.1.3. HLC LEVEL SETUP

| 1. HLC LEVEL | LOW/HIGH |
| --- | --- |
| 2. MASK COLOR | 0~10 |
| 3. EXIT |  |

■ HLC LEVEL

Sensitivity is adjustable.

■ MASK COLOR

Masking color on the high light is adjustable.

2.2. MOTION DETECT

| 1. MD NUM : | 1~8 |
| --- | --- |
| 2. MD DEFINE : | OFF/ON |
| 3. MD DISPLAY : | OFF/ON |
| 4. AREA DISPLAY | OFF/ON |
| 5. SENSITIVITY | LOW/HIGHT |
| 6. AREA TOP | 1~26 |
| 7. ARA BOTTOM | 5~30 |
| 8. ARA LEFT | 1~44 |
| 9. AREA RIGHT | 5~48 |
| 10.EXIT |  |

If MD DISPLAY is set to ON, “MOTION” appears

on the bottom right.

Motion Detection doesn’t work when

OSD MENU is activated.

■ MD DEFINE

When DEFINE is ON, MD AREA can be set up

to 8.

■ SENSITIVITY

Sensitivity of Motion Detection is adjustable.

If the sensitivity gets higher, there is a possibility

to have incorrect workings.

<Caution>

- Motion Detection doesn’t work in

SCAN & PATTERN setting.

- When user uses MD, DWELL TIME should

be set over 3 seconds at least in TOUR setting.

- If there is a shaken lighting directly, do not

set the MD because there may be an

incorrect working.

- If there is an instant change for lights which

has an effect on the subject, an incorrect

working may occurs.

- This MD function is for neither fire protection

nor antitheft so that provider is free from

any responsibilities of the malfunction.

2.3. ATW

| 1. ATW MODE: | AUTO/MANUAL/<HOLD>/OUT DR/IN DR |
| --- | --- |
| 2. MANUAL RED: | 0 ~ 200 |
| 3. MANUAL BLUE: | 0 ~ 200 |
| 4. EXIT |  |

Menu Shift : UP, DOWN, LEFT, RIGHT KEY

Menu Open : IRIS OPEN KEY

Menu Close : IRIS CLOSE KEY

It is set to White Balance under any lighting

conditions.

■ ATW MODE  ATW

White Balance is compensated automatically.

■ ATW MODE  < HOLD >

White Balance will be optimized under the

current light condition. If the light condition is

changed, White Balance should be readjusted.

It works by pressing IRIS OPEN KEY when

< HOLD > is off.

■ ATW MODE  OUT DR

White Balance will be optimized under the

outdoor environment.

■ ATW MODE  IN DR

White Balance will be optimized under the

indoor environment.

■ ATW MODE  MANUAL

White Balance is adjustable by RED and BLUE

gains manually.

<Caution>

1. When it is too dark or bright on the background,

White Balance may not function properly.

2. At the place the camera is directed towards

the fluorescent light or lighting conditions are

changeable, White Balance may work improperly.

2.4. FOCUS/ZOOM

| 1. FOUCS MODE: | MANUAL/AUTO/ZMTRG |
| --- | --- |
| 2. ZOOM TRK MODE: | ON/OFF |
| 3. ZOOM TRK ZPEED: | FAST/SLOW |
| 4. DIGITAL ZOOM: | OFF/ON |
| 5. ZOOM MAG: | 2X/4X/6X/8X/10X/12X |
| 6. ZOOM SPEED | 1~8 |
| 7. EXIT |  |

■ FOCUS MODE

* MANUAL

FOCUS is adjustable manually.

* AUTO

FOCUS is adjustable automatically.

* ZMTRG

Whenever ZOOM is changed, FOCUS is also

readjusted.

■ ZOOM TRK MODE

When ZOOM is changed in Wide <-> Tele,

user can select if FOCUS will be worked or not.

It is applied only to AUTO and ZOOM TRK mode.

When user moves ZOOM to a specific area and

wants to get faster, ZOOM TRK MODE should

be set to Off.

<Caution>

Under the following conditions, AUTO FOCUS may

not work properly.

1. When the lux level is too low or too high

2 .When a short distance object and a close

distance object appear together in a monitoring area.

3. When there is no contrast between Black and White.

2.5. AE

| 1. BRIGHTNESS: | 0 ~ 100 |
| --- | --- |
| 2. IRIS: | AUTO/MANUAL |
| 3. IRIS MANUAL: | F1.6 ~ CLOSE |
| 4. SHUTTER: | ESC/MANUAL/A.FLK |
| 5. SHUTTER MANUAL: | X256~1/120000 |
| 6.AGC: | OFF/LOW/MID/HIGH |
| 7. SSNR: | OFF/LOW/MID/HIGH |
| 8. DSS: | X2~X256, OFF |
| 9. EXIT |  |

■ BRIGHTNESS

Brightness on the screen is adjustable.

■ IRIS

The brightness on the screen can be

adjustable depending on the amount of light

through exposure meter.

AUTO : The exposure meter is adjusted automatically.

MANUAL : User can adjust the exposure meter

manually to optimize the brightness

on the screen.

■ SHUTTER

When capturing a subject, moving fast or under low

light condition, user can adjust the shutter speed

to faster or slower to get improved images.

ESC  : Shutter speed is controlled automatically

according to the brightness on the screen.

MANUAL : Shutter speed is controlled in

1/20000 ~ X128 manually.

- FLK : Use this mode when the screen flickers

due to the inconsistent frequency under

surrounding lights.

In this mode, shutter speed fixed at 1/100 sec.

In NTSC system and 1/120 sec. in PAL system.

■ SHUTTER MANUAL

Shutter speed is controllable only when SHUTTER

is set to MANUAL.

■ AGC

Auto Gain Control is to brighten the screen under

the low light condition.

■ SSNR ( DIGITAL NOISE REDUCTION )

Noise is lessened under the low light condition.

OFF : Noise reduction has no effect.

LOW: Noise reduction effect is small but there is

little afterimage.

MID: Noise reduction effect generally goes proper

and afterimage is not strong.

HIGH: Noise reduction effect is excellent but

afterimage is also strong.

■ DSS

By controlling Electronic Shutter, it makes the

screen brightened under the low light condition.

However, if user sets higher ratio of DSS, afterimage

becomes stronger and Auto Focus may not work.

<Caution>

1. If possible, do not use A.FLK and WDR mode at

the same time.

2. In internal synchronization system, if SHUTTER is

set to ESC and the camera faces to sunlight or

strong light reflecting areas directly, the screen

goes wrong. In this case, change the location of

the camera.

3. In case that SHUTTER is set to MANUAL or A.FLK,

SENSUP LIMIT doesn’t work.

4. SSNR doesn’t work when AGC is OFF.

2.6. DAY&NIGHT

| 1. D/N MODE: | COLOR/BW/AUTO |
| --- | --- |
| 2. BURST MODE: | OFF/ON |
| 3. BURST LEVEL: | 0 ~ 100 |
| 4.DURATION: | FAST/SLOW |
| 5. FILTER DELAY: | 5/7/10/15/20/30/40/60SEC |
| 6. EXIT |  |

B/W mode with removed IR cut filter offers

crystallized clear image under low light condition.

On the contrary, Color mode with replaced

IR cut filter works at daytime.

■ D/N MODE

AUTO : Color mode at daytime and B/W mode

at nighttime. It is switchable automatically

according to the designated lux condition.

■ BURST MODE

OFF : BURST signal is removed.

ON : BURST signal is output with together B/W

Luminance signal.

■ BURST LEVEL

BURST LEVEL is adjustable when D/N MODE is

COLOR.

■ DURATION

Switching over lux condition is selectable in

DAY<->NIGHT

|  | DAY->NIGHT | NIGHT->DAY |
| --- | --- | --- |
| FAST | 2.5LUX | 4LUX |
| SLOW | 0.8LUX | 6LUX |

■ FILTER DELAY

The duration of Dwell Time of both the lighting

conditions can be customized to let the camera

divert between daytime and nighttime settings.

<Caution>

- When AGC is OFF, AUTO MODE doesn’t

work while COLOR or B/W mode works.

- If sunlight or halogen lamp is used in

B/W mode, Focus may be blurred or dimmed.

2.7. SPECIAL

| 1. H-REV: | OFF/ON |
| --- | --- |
| 2. V-REV: | OFF/ON |
| 3. DIS: | OFF/ON |
| 4. SHARPNESS: | 0~100 |
| 5. COLOR: | 0~100 |
| 6. EXIT |  |

■ H-REV

The screen flips horizontally.

■ V-REV

The screen flips vertically.

■ DIS

Digital Image Stabilizer compensates the screen

from an unstable environment which can move

the camera.

■ SHARPNESS

Overall sharpness on the screen is adjustable.

■ COLOR

Overall color density on the screen is adjustable.

3. AUTOSEQ SET

| 1. PRESET | <SET> |
| --- | --- |
| 2. TOUR | <SET> |
| 3. SCAN | <SET> |
| 4. PATTERN 5. AUTO PAN | <SET> <SET> |
| 6. AUTO MODE SET 7. HOME POSITION 8. EXIT | OFF/SEQ/TOUR/SCAN/ PAN/A.PAN OFF/1~128 |

Menu Shift : UP, DOWN, LEFT, RIGHT KEY

Menu Open : IRIS OPEN KEY

Menu Close : IRIS CLOSE KEY

PRESET : Location of the current PAN/TILT and

ZOOM can be programmed. In ALARM

or TOUR operation, the camera moves

to the programmed preset.

SEQ   : The programmed PRESET is executed

from the lower order successively.

TOUR : PRESET works in GROUP. Up to 5

presets per group can be registered and up

to 5 groups can be operated.

SCAN : PAN movers from the starting point

and the ending point and up to 5 scans

can be set up.

PATTERN : The operation path of PAN/TILT is saved

and PATTERN plays PAN/TITL according

to the operation path.

■ AUTO MODE SET

OFF :  It keeps on Operation Standby condition.

TOUR, SCAN, PATT : They will be executed

automatically after user exits OSD MENU.

■ HOMEPOSITION

If there is any loss of power the camera goes to the

programmed Preset.

<Caution>

If AUTORUN is set the camera does not go to the

programmed Preset.

3.1. PRESET

This function is to move the designated position

if user selects the desirable target and location.

| 1. PRESET NO: | 1~128 |
| --- | --- |
| 2. PRESET DEFINE: | OFF/<ON> |
| 3. PRESET NAME: | OFF/<ON> |
| 4. P/T POSITION: | P/T ANGLE |
| 5. BRIGHTNESS: | 0~100 |
| 6. ATW: | <SET> |
| 7. BLC: | OFF/<WDR>/<USER>/<HLC> |
| 8. EXIT |  |

Menu Shift : UP, DOWN, LEFT, RIGHT KEY

Menu Open : IRIS OPEN KEY

Menu Close : IRIS CLOSE KEY

■ PRESET NO

Total 128 presets can be set up.

Change of PRESET NO is done by

LEFT and RIGHT KEY.

■ PRESET DEFINE

OFF : PRESET doesn’t work

ON : Applicable PRESET is workable.

Location change of PAN/TILT/ZOOM in

PRESET is as follows.

- After PRESET DEFINE is changed from

OFF to ON, press IRIS OPEN KEY

- Locate PAN/TILT to the desirable place.

PREST NAME: PRESET0001

PAN: XXX

TILT: XXX

ZOOM: T/W EXIT: IRIS CLOSE

- If user wants to the location of ZOOM, press

TELE or WIDE button of controller.

- Finalize FAN/TILT and ZOOM settings by

pressing IRIS CLOSE KEY.

■ PRESET NAME

OFF : After PRESET works, PRESET name is not

displayed on the screen.

<ON> : After PRESET works, PRESET name is

displayed on the screen.

Default of PRESET name is PRESET0001 and the

method of editing is as follows.

- After changing PRESET NAME from OFF to

<ON> and press IRIS OPEN KEY.

- How to edit PRESET NAME is the same

as “1.2 CAMERA NAME DISPLAY”.

■ BRIGHTNESS

The brightness of the current PRESET is adjustable.

■ ATW

The White Balance of current PRESET is

selectable. The setting is as follows.

1. Select ATW menu

2. Press IRIS OPEN KEY in <SET>.

3. How to set up ATW is the same as “2.3. ATW”.

■ BLC

When the subject gets darker under the bright

background in the current PRESET, this function

is useful.

How to set up is as follows.

1. Select BLC menu.

2. Press IRIS OPEN KEY in <SET>.

3. How to set up ATW is the same as “2.1. BLC”

<Caution>

- When setting PRESET by the shortcut key,

PRESET can be set in No.1-No.64 and,

regarding over No.65, it should be set up

by OSD Menu.

- Default set up value of BRIGHT, BLC and

ATW in PRESET is decided by Factory Default.

3.2. TOUR

This function is to repeatedly operate

Group Surveillance with programmed presets

| 1. TOUR NO: | 1 ~ 5 |
| --- | --- |
| 2. TOUR NO DEF: | OFF/<ON> |
| 3. NAME: | TOURGROUP1 |
| 4. TOUR NAME DEF: | OFF/<ON> |
| 5. DWELL TIME: | 1~120 |
| 6. EXIT |  |

■ TOUR NO

Maximum 5 groups can be registered.

■ TOUR NO DEF

OFF : TOUR Group setting is cancelled.

ON : TOUR Group is set up.

<Setting example>

| TOUR NO | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| TOUR DEF | ON | OFF | ON | OFF | ON |

TOUR will be executed in programmed preset 1, 3 & 5.

How to set TOUR GROUP 1 in PRESET is as follows.

1. Register “1” as a TOUR NO.

2. After changing TOUR NO DEF from OFF

to <ON>, press IRIS OPEN KEY.

| 1. PRESET NUMBER | OFF/1~128 |
| --- | --- |
| 2. PRESET NUMBER | OFF/1~128 |
| 3. PRESET NUMBER | OFF/1~128 |
| 4. PRESET NUMBER | OFF/1~128 |
| 5. PRESET NUMBER | OFF/1~128 |

3. Input desirable PRESET No. from 1 in serial order.

4. Press IRIS CLOSE KEY to finalize the setting.

■ NAME

It changes the name of TOUR GOURP,.

How to change the name is as follows.

- After TOUR NAME DEF changes from OFF to

<ON>, press IRIS OPEN KEY.

- How to change the Tour group name is the same

as “1.2 CAMERA NAME DISPLAY”.

■ DWELL TIME

It is a waiting time between PRESET execution and

next PRESET.

<Caution>

1. If TOUR GOURP1 PRESET is set up as follows.

| 1. PRESET NUMBER | 1 |
| --- | --- |
| 2. PRESET NUMBER | 2 |
| 3. PRESET NUMBER | 3 |
| 4. PRESET NUMBER | OFF |
| 5. PRESET NUMBER | 4 |

After operating as

PRESET1 -> PRESET2 -> PRESET3, it is returned

to PRESET1.

Namely, if there is “OFF” during the operation in

serial order, it is returned to the first preset

3.3. SCAN

This function is to surveil the designated two

points in the designated speed by automatic

travel back and forth.

| 1. SCAN NO | 1 ~ 5 |
| --- | --- |
| 2. SCAN DEFINE | OFF/ON |
| 3. NAME | SCANGROUP1 |
| 4. SCAN NAME DEF | OFF/<ON> |
| 5. PAN START POS | <180> |
| 6. PAN END POS | <350> |
| 7. TILT POS | <45> |
| 8.SCAN SPEED | 5°,10°,15°,20°/S |
| 9. EXIT |  |

■ SCAN NO

Maximum 5 Scans can be registered.

■ SCAN NO DEF

OFF : SCAN is cancelled

ON : SCAN can be set up (it should be

“ON” for the operation in AUTO RUN)

<Setting example>

| SCAN NO | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| SCAN DEF | ON | OFF | ON | OFF | ON |

SCAN will be executed in programmed preset 1,3 &5.

How to set up SCAN1 in PRESET is as follows.

1. Register “1” as s SCAN NO.

2. After changing SCAN DEFINE from OFF to

<ON>, press IRIS OPEN KEY.

■ NAME

It changes SCAN Name.

How to change is as follows.

- After changing SCAN NAME DEF from OFF to

<ON>, press IRIS OPEN KEY.

- How to change the SCAN name is the same as

“1.2 CAMERA NAME DISPLAY”.

■PAN START POS

It sets up the initialized location of SCAN.

Controllable angle is 0~ 360° , 0 ~ -360°.

How to set up is as follows.

1. Press IRIS OPEN KEY in PAN START POS.

PAN START POSITION SET

PAN START POSITION: XXX

EXIT: IRIS CLOSE

PAN Shift : LEFT, RIGHT KEY

2. After shifting to a desirable position, press

IRIS OPEN KEY.

■ PAN END POS

It sets up the terminated location of SCAN.

Controllable angle is 0°~ 360°, 0°~-360°.

How to set up PAN END POS is the same as

“PAN START POS”.

■ TILT POS

It sets up TILT angel in SCAN.

Controllable angle is 0~ 90° .

How to set up is the same as “PAN START POS”

but UP, DOWN KEY should be used for TILT

operation.

■ SCAN SPEED

It sets up SCAN Speed..The speed can be set up

in 5°/S(Slow) ~ 20°/S(Fast).

<Caution>

The value of Brightness, WDR, ATW is applied

to the setting values on the OSD.

3.4. PATTERN

It is to memorize the paths operated with the

joystick operation and reenact them..

| 1. PATT NUMBER | 1 ~ 2 |
| --- | --- |
| 2. PATT DEFINE | OFF/ON |
| 3.NAME | PATTERN001 |
| 4.PATT NAME DEF | OFF/<ON> |
| 5.PATT RECORD | OFF/<ON> |
| 6.PATT SPEED | 5°,10°,15°,20°/S |
| 7. EXIT |  |

MENU Shift : UP, DOWN, LEFT, RIGHT KEY

MENU Open : IRIS OPEN KEY

MENU Close : IRIS CLOSE KEY

■ PATT DEFINE

OFF : PATTERN is cancelled

ON : PATTERN is set up.

If operating it in AUTO RUN, PATT DEFINE should

to be ON.

■ NAME

It changes PATTERN Name..

How to set up is as follows..

1. After changing PATTERN NAME DEF from

OFF to <ON>, press IRIS OPEN KEY.

2. How to change the name is the same as

“1.2 CAMERA NAME DISPLAY”.

■ PATT RECORD

OFF :  PATTERN is not memorized.

ON  : PATTERN is memorized.

How to set up PATTERN

1. After PATT RECORD changes from OFF to

<ON>, press IRIS OPEN KEY.

MEMORY FILL: XXX ( memory storage )

START:IRIS OPEN

EXIT:IRIS CLOSE

PAN/TILT Shift :UP,DOWN,LEFT,RIGHT

2. Move PAN/TILT to the desirable position

before starting PATTERN.

3. Press IRIS OPEN KEY to start the process

4. Press IRIS CLOSE to save the process.

< Caution>

1. If there is no PAN/TILT movement, no PATTERN

is memorized.

2. The value of Brightness, WDR, ATW is applied

to the setting values on the OSD.

3.5. AUTO PAN

Pan is rotate 360° according to setting value

of TILT ANGLE, DIRECTION and SPEED

| 1. TILT ANGLE | <XXX> |
| --- | --- |
| 2. PAN SPEED | 2°,5°,10°,20°/S |
| 3. DIRECTION | CW/CCW |
| 4. EXIT |  |

■ TILT ANGLE

Move to TILT ANGLE and Press MANU KEY.

For FINISH, Press MANU KEY.

<Caution>

- Zoom is automatically switched to 1X.

- WDR, ATW are set to FACTORY DEFAULT.

- If set an AUTO PAN at TILT over 90°,

PAN/TILT automatically moves to the

position -20°~90° and TILT is limited to 90°.

4. ZONE SET

| 1.AREA SEL | 1 ~ 8 |
| --- | --- |
| 2.AREA DEFINE | OFF/ON |
| 3.AREA NEW SET | OFF/ON |
| 4.TOP/BOTTOM-1 | XXX |
| 5.TOP/BOTTOM-2 | XXX |
| 6.LEFT/RIGHT-1 | XXX |
| 7.LEFT/RIGHT-2 | XXX |
| 8.EXIT |  |

MENU Shift : UP, DOWN, LEFT, RIGHT KEY

MENU Open : IRIS OPEN KEY

MENU Close : IRIS CLOSE KEY

■ AREA SEL

Up to 8 PRIVACY AREAs can be set up.

■ AREA DEFINE

OFF : PRIVACY ZONE is not activated..

ON : PRIVACY ZONE is activated.

■ AREA NEW SET

OFF : Previous PRVIACY ZONE is activated.

ON : New PRIVACY ZONE is set up from the

current PAN/TILT position.

(When it switches from OFF to ON, PRIVACY

ZONE appears in the center on the screen.)

* How to set up PRIVACY ZONE1*

1. When the MENU is OFF, locate

PRIVACY ZONE block in the center on the

screen by using PAN/TILT function.

2. Press IRIS OPEN KEY to activate MENU and

move to “ZONE SET”. And then, press IRIS

OPEN KEY again.

3. Change AREA SEL to 1.

4. Change AREA DEFINE from OFF to ON.

5. Change AREA NEW SET from OFF to ON.

6. New ZONE is activated in the center on the

screen and the ZONE can be moved by

TOP, BOTTOM, LEFT, RIGHT key.

7. Press IRIS CLOSE KEY to finalize the set up.

5. ALARM SET

| 1. ALARM DISPLAY | OFF/ON |
| --- | --- |
| 2. ALARM IN | <SET> |
| 4. ALARM OUT | OFF/ON |
| 5. TIME OUT | 1 ~ 10Sec |
| 6. EXIT |  |

■ ALARM DISPLAY

When signals are input from outside, the alarm

number is displayed on the monitor.

■ ALARM IN

Move to the PRESET by detection of

ALARM INPUT from external sensor.

*How to set up PRESET

- Move to ALARM IN, Press IRIS OPEN KEY.

Below menu will follow.

| 1.IN1 PRESET NUM | OFF/1 ~128 |
| --- | --- |
| 2.IN2 PRESET NUM | OFF/1 ~128 |
| 3.IN3 PRESET NUM | OFF/1 ~128 |
| 4.IN4 PRESET NUM | OFF/1 ~128 |
| 5.EXIT |  |

- Set a PRESET number.

- Move to EXIT and Press RIGHT KEY to

finalize the set up.

■ TIME OUT

Set a TIME OUT for waiting time from PRESET

to reactivation of TOUR after ALARM IN.

<Caution>

- PRESET DEFINE should be set to “ON” to

activate “ALARM IN”

A ground of external alarm signal should be

maintained at least 200ms

6. INITIALIZE SET

1. POWER ON RESET

2. PAN/TILT INIT

3. CAMERA INIT

4. AUTO SEQ INIT

5. PRIVACY ZONE INIT

6. FACTORY INIT

7. EXIT

MENU Shift : UP, DOWN

MENU Open : IRIS OPEN KEY

MENU Close : IRIS CLOSE KEY

■ POWER ON RESET

The unit is initialized.

■ PAN/TILT INIT

1. PAN/TILT MENU is initialized.

2. PAN/TILT location is initialized.

■ CAMERA INIT

Only CAMERA setting menu is initialized..

■ AUTO SEQ INIT

1. Only AUTO SEQ menu is initialized.

2. Setting angle of PRESET and Memory of

PATTERN are not initialized.

■ PRIVACY ZONE INIT

PRIVACY ZONE is initialized.

■ FACTORY INIT

All set ups are initialized as a factory default.

※ WTX-1200A – Simple PRESET Setting ※

It works when OSD MENU is not activated.

e.g.> How to set up PRESET 1

1. Place PAN/TILT/ZOOM to a desirable place

by Joystick.

2. Press Keypad No. 1.

3. Press F1 Key till “SET OK” appears in the bottom

right on the screen.

<Caution>

When setting PRESET by the shortcut key,

PRESET can be set in No.1-No.64 and, regarding

over No.65, it should be set up by OSD Menu.

※ WTX-1200A – Simple PRESET Shift ※

It works when OSD MENU is not activated.

Press F1 key shortly after pressing RRESET

number. And then, it moves to the

corresponding PRESET. However, it is valid only

up to PRESET no. 32 of which PRESET DEFINE

is set to ON.

※WTX-1200A - Simple TOUR operation※

It works when OSD MENU is not activated

Press F1 key shortly after pressing Keypad No. 71.

It works when TOUR DEFINE is ON and TOUR

related MENU is set up properly.

※WTX-1200A - Simple PATTERN operation ※

It works when OSD MENU is not activated

Press F1 key shortly after pressing Keypad No. 66.

It works when PATTERN DEFINE is ON and

PATTERN related MENU is set up properly.

※ WTX-1200A - Simple SCAN operation ※

It works when OSD MENU is not activated

Press F1 key shortly after pressing Keypad No. 81.

It works when SCAN DEFINE is ON and SCAN

related MENU is set up properly.

※ WTX-1200A - Simple Sequency operation ※

It works when OSD MENU is not activated

Press F1 key shortly after pressing Keypad No. 70.

It works when PRESET is set up.

※ Quick Operation Key Table ※

[PELCO D /P PROTOCOLS]

| Number | Function |
| --- | --- |
| 1~64+Preset | Setting Preset Press Preset button over 2 seconds. |
| 1~64+Preset | Executing Preset |
| 66+Preset | Executing Scan |
| 70+Preset | Executing Sequency |
| 71+Preset | Executing Tour |
| 81+Preset | Executing Pattern |
| 95+Preset | IRIS OPEN (OSD Menu ) |
| 96+Preset | IRIS CLOSE |

6. Produce Structure(mm)

- Specification

| Model | 27x/ 33x/ 37x Speed Domes |
| --- | --- |
| Image sensor | 1/4” Super HAD CCD(27x)/ Color Vertical Double Density CCD(33x, 37x) |
| Effective Pixels | NTSC : 768(H) x 494(V), PAL : 752(H) x 582(V) |
| H. Resolutions | 550TVL(Color)/ 680TVL(B/W) |
| Synchronizing system | Internal |
| Scanning system | 2:1 Interlaced |
| Video output | VBS : 1.0Vp-p Composite, 75 ohms |
| S/N Ratio | More than 50dB(AGC off) |
| Min. illumination | 0.7Lux/F1.6(Color), 0.0002Lux(Sens-ups, x256), 0.06Lux/F1.6(B/W) |
| OSD | On/ Off |
| Digital zoom | On/ Off(x2, x4, x8, x10, x12) |
| Day & Night | Color/ BW/ AUTO(ICR) |
| Back Light Compensation | WDR(for 33x, 37x only)/ BLC/ HLC/ Off |
| Shutter Speed | Auto/ Manual (NTSC : 1/60~1/120,000 sec., PAL : 1/50 ~ 1/120,000 sec.) |
| White Balance | AWD/ Manual/ ATW(1,800°K ~ 10,500°K) |
| Gain Control | Low/ Middle/ High/ Off |
| Sens-up | On/ Off(2x ~ 256x) |
| 3D Noise Filter | Low/ Middle/ High/ Off |
| Privacy | On/ Off(8 Programmable zones) |
| Preset | 256 Points |
| Digital Flip | On/ Off |
| Focus control | Auto/ Manual/ One Push |
| Iris control | Auto/ Manual |
| Motion Detection | On/ Off(Built-in Alarm output connector) |
| Protocol | RS-485(PELCO-D/P) |
| Alarm IN/OUT | 3/1 |
| Power Source | 24VAC / 2A |
| Operating current | 35W Max.(Heater on) |
| Lens | Auto Focus Digital zoom |
| Aperture ratio | 37x : F1.6~F3.9, 33x : F1.6~F3.6, 27x : F1.6~F2.9 |
| Pan/Tilt angle | Pan : 360°(400°/sec.), Tilt : 0°~180°(400°/sec.) |
| Operating temperature | -40°F~122°F(-40℃ ~ +50℃) |
| Humidity | Within 90% RH |
| Measurement(mm) | Outdoor : 255(Ø) x 377(H) |
| Weight(Approx.g) | 4,500(Outdoor) |